# Report on

Simulation of V-I Characteristics of 1N4007 Silicon Diode

For the fulfilment of Event-2 in Elements of Electronics Engineering (24CB240)

Submitted by

| Sl. No. | Name of the student | USN | Marks [20M] |
| --- | --- | --- | --- |
| 1. | Akshith Thomas | 01JST25UCB009 |  |

Submitted to

Prof. Shalini M S
Assistant Professor
Department of ECE,
JSS Science and Technology University, SJCE, Mysuru

DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING
2025-2026

## Department of Electronics and Communication Engineering

Rubrics for event evaluation (Even Sem- Year-2025-26)
Subject/Code No: Elements of Electronics Engineering -24EC211
Semester/Section: II Semester

Rubrics for an Event

| Criterion | Marks Allotted | Marks Obtained |
| --- | --- | --- |
| Problem Analysis and Solution | 4M |  |
| Simulation Implementation | 4M |  |
| Results & Interpretation | 4M |  |
| Presentation & Communication | 4M |  |
| Report | 4M |  |
| Total | 20M |  |

Signature of Faculty: ___________________

## Experiment: Simulation of V-I Characteristics of a 1N4007 Silicon Diode

### Aim:

To design, simulate, and verify the Forward and Reverse Voltage-Current (V-I) characteristics of a 1N4007 Silicon Diode using a custom Web-Based Interactive Circuit Simulator and plot its knee voltage, forward voltage drop, and reverse leakage current.

### Components Required:

The following virtual components were used in the simulation environment: * DC Voltage Source (Variable: 0 to 1.2V for Forward, 0 to -100V for Reverse) * Resistor (1kΩ) * 1N4007 Silicon Diode * Ammeter and Voltmeter (Digital Readouts) * Interactive V-I Cartesian Graph Plotter

### Theory:

A P-N junction diode is a semiconductor device that allows current to flow in one direction (forward bias) and heavily restricts it in the opposite direction (reverse bias).

- Forward Bias: When the P-type material is connected to the positive terminal and N-type to the negative terminal, the depletion region narrows. Once the applied voltage exceeds the barrier potential (Knee Voltage, Vk≈0.7V for silicon), the diode conducts heavily, and current increases exponentially.

- Reverse Bias: When the polarity is reversed, the depletion region widens, acting as an insulator. Only a negligible minority-carrier current flows, known as the reverse leakage current (in the range of microamperes, μA), until the breakdown voltage is reached (not simulated, as 1N4007 breakdown is 1000V).

### Block diagram / Circuit Configuration:

(Below is the circuit configuration utilized in the simulation)

Circuit Diagram

### Mathematical Modeling (Shockley Diode Equation):

The ideal diode equation approximates the V-I characteristics:

ID=IS×eVDnVT−1

Where: * ID = Diode Current * IS = Reverse saturation current (≈−0.05μA) * VD = Voltage across diode * VT = Thermal voltage (≈26mV at room temp)

### Procedure:

- Initialize the interactive web simulation environment.

- Forward Bias Test:

  - Toggle the simulation to “Forward Bias” mode.

  - Slowly sweep the input voltage from 0V to 1.2V using the slider control.

  - Observe the current at 0V→0.69V.

  - Note the point at which current begins to exponentially rise (Knee Voltage).

  - Record the forward current (If) at maximum forward voltage.

- Reverse Bias Test:

  - Toggle the simulation to “Reverse Bias” mode.

  - Sweep the input voltage from 0V to −100V.

  - Observe the negligible current flow and confirm the reverse leakage current floor.

- Observe the V-I plot tracing simultaneously with the voltage adjustments.

### Observation:

The simulation accurately demonstrated the nonlinear V-I properties of the 1N4007.

Forward Characteristics: * From 0V to 0.69V, the diode remained in a non-conducting state, with 0mA current flowing. * At exactly 0.7V (Knee Voltage), the visual circuit indicator triggered, and current began flowing. * Beyond 0.7V, the current increased exponentially, reaching approximately 1000mA at 1.0V.

Forward Bias

Reverse Characteristics: * The current was entirely blocked by the diode, reading a microscopic negative value of exactly −0.05μA. * This value remained perfectly flat and constant from 0V all the way to −100V, demonstrating the theoretical ideal reverse leakage behavior.

Reverse Bias

### Result:

The V-I characteristic curves of the 1N4007 silicon diode were successfully simulated and plotted using the interactive web tool. * Knee Voltage Observed: 0.7V * Reverse Leakage Current Observed: ≈−0.05μA * Forward Drop: Conducts heavily beyond 0.7V.

Final View

### Inference:

- The diode operates exactly as an electronic one-way valve.

- The non-linear behavior proves that standard Ohm’s law (V=IR) does not apply directly to semiconductor devices without considering the barrier potential.

- The software perfectly replicated the Shockley diode equation, confirming theoretical textbook models against visual simulation data.

### Conclusion:

The objective of simulating and plotting the V-I characteristics of a 1N4007 silicon diode was successfully achieved using an interactive web simulator. The plotted knee voltage, forward exponential curve, and reverse leakage current precisely matched the theoretical characteristics of a real-world silicon P-N junction diode.